Team3 Textile Industrial

Firewall Policy & Live Demo Script

Purdue Model Network Segmentation — pfSense

PART 1 — Firewall Policy Overview

Each VLAN interface on pfSense has specific rules controlling which machines can communicate and on which ports. Rules are read top to bottom — first match wins.

# VLAN10 — Level 1: PLC Network (10.10.10.0/24)

Contains: PLC-SIM01 (10.10.10.10), HMI-DESKTOP (10.10.10.12). Most restricted level — no internet, no IT access.

| Rule | Source | Destination | Port | Action |
| --- | --- | --- | --- | --- |
| 1 | VLAN10 (any) | VLAN10 (any) | 502 | ✓ PASS — Intra-VLAN Modbus |
| 2 | VLAN10 (any) | 10.10.20.10 (IOT-GW01) | 502 | ✓ PASS — Modbus to IoT GW |
| 3 | VLAN10 (any) | Any | Any | ✗ BLOCK — All other traffic |

Why: PLCs only need Modbus. No internet = no external attack surface.

# VLAN20 — Level 2: IoT Gateway (10.10.20.0/24)

Contains: IOT-GW01 (10.10.20.10). Reads PLC via Modbus, runs Mosquitto MQTT broker.

| Rule | Source | Destination | Port | Action |
| --- | --- | --- | --- | --- |
| 1 | VLAN20 (any) | 10.10.10.10 (PLC-SIM01) | 502 | ✓ PASS — Modbus read from PLC |
| 2 | VLAN20 (any) | Any | Any | ✗ BLOCK — All other traffic |

Why: IoT gateway only polls the PLC. No internet prevents data exfiltration.

# VLAN30 — Level 3: OT Operations (10.10.30.0/24)

Contains: WIN-ENG01 (10.10.30.10), HISTORIAN01 (10.10.30.20), SCADA01 (10.10.30.30).

| Rule | Source | Destination | Port | Action |
| --- | --- | --- | --- | --- |
| 1 | VLAN30 subnets | pfSense LAN | 443 | ✓ PASS — pfSense GUI management |
| 2 | 10.10.30.20 | 10.10.20.10 (IOT-GW01) | 1883 | ✓ PASS — MQTT subscribe |
| 3 | 10.10.30.10 (WIN-ENG01) | Any | Any | ✓ PASS — Internet for management |
| 4 | VLAN30 subnets | 10.10.40.20 (Wazuh) | 1514 | ✓ PASS — Wazuh agent |
| 5 | 10.10.30.20 (Historian) | VLAN10 subnets | Any | ✗ BLOCK — Historian cannot reach PLC |
| 6 | 10.10.30.20 (Historian) | Any | Any | ✓ PASS — Historian internet |
| 7 | VLAN30 subnets | Any | Any | ✗ BLOCK — All other traffic |

Why: Historian subscribes to MQTT but is explicitly blocked from reaching PLC directly.

# VLAN35 — Level 3.5: DMZ (10.10.35.0/24)

Contains: WEB-DMZ01 (10.10.35.10), REVERSE-PROXY (10.10.35.20). Public-facing, isolated from OT/IT.

| Rule | Source | Destination | Port | Action |
| --- | --- | --- | --- | --- |
| 1 | VLAN35 subnets | 10.10.40.20 (Wazuh) | 1514 | ✓ PASS — Wazuh agent |
| 2 | VLAN35 subnets | VLAN10 subnets | Any | ✗ BLOCK — DMZ cannot reach PLC |
| 3 | VLAN35 subnets | VLAN20 subnets | Any | ✗ BLOCK — DMZ cannot reach IoT |
| 4 | VLAN35 subnets | VLAN40 subnets | Any | ✗ BLOCK — DMZ cannot reach IT |
| 5 | VLAN35 subnets | VLAN30 subnets | Any | ✗ BLOCK — DMZ cannot reach OT |
| 6 | VLAN35 subnets | Any | Any | ✓ PASS — Internet access |

Why: If web server is compromised, attacker cannot pivot to OT or IT networks.

# VLAN40 — Level 4: Enterprise IT (10.10.40.0/24)

Contains: DC01, WAZUH01, WIN-USER01/02, WIN-ADMIN01. Corporate IT with internet.

| Rule | Source | Destination | Port | Action |
| --- | --- | --- | --- | --- |
| 1 | VLAN40 subnets | 10.10.30.20 (Historian) | 3000 | ✓ PASS — Grafana dashboard |
| 2 | VLAN40 subnets | VLAN35 subnets | 80 | ✓ PASS — Company website |
| 3 | VLAN40 subnets | VLAN40 subnets | 443 | ✓ PASS — Wazuh dashboard |
| 4 | VLAN40 subnets | VLAN10 subnets | Any | ✗ BLOCK — IT cannot reach PLC |
| 5 | VLAN40 subnets | VLAN20 subnets | Any | ✗ BLOCK — IT cannot reach IoT |
| 6 | VLAN40 subnets | Any | Any | ✓ PASS — Internet access |

Why: IT reads OT data via Grafana only. Direct PLC/IoT access is blocked.

# WAN — Inbound Rules

| Rule | Source | Destination | Port | Action |
| --- | --- | --- | --- | --- |
| 1 | Any (internet) | 10.10.35.20 (Reverse Proxy) | 80 | ✓ PASS — HTTP to website |
| 2 | Any (internet) | WAN address | 1194 | ✓ PASS — OpenVPN remote access |
| 3 | Reserved/Bogon | Any | Any | ✗ BLOCK — Bogon networks |

PART 2 — Live Demo (All Levels)

For each level we run one allowed test and one blocked test to prove the policy is enforced.

# Opening Statement

"I have just shown you our full firewall policy. Now I will demonstrate it live — for each level I will show what is allowed and what is blocked."

# Demo — Level 1: PLC Network

## What to say:

"PLC-SIM01 is on Level 1 — the most restricted level. It should only communicate via Modbus. Watch what happens when it tries to reach the internet."

## Machine: PLC-SIM01 (10.10.10.10) — Terminal (SSH)

| What we test | Command | Expected | Result |
| --- | --- | --- | --- |
| Modbus port open (HMI → PLC) | nc -zv -w 3 10.10.10.10 502 | succeeded | ✓ PASS |
| PLC cannot reach internet | ping -c 3 8.8.8.8 | 100% packet loss | ✗ BLOCKED |

Level 1 is completely isolated — Modbus works, internet is blocked.

# Demo — Level 2: IoT Gateway

## What to say:

"IOT-GW01 is on Level 2. It reads PLC data via Modbus and publishes to MQTT. It should have no internet access."

## Machine: IOT-GW01 (10.10.20.10) — Terminal (SSH)

| What we test | Command | Expected | Result |
| --- | --- | --- | --- |
| IoT GW can read PLC via Modbus | nc -zv -w 3 10.10.10.10 502 | succeeded | ✓ PASS |
| IoT GW cannot reach internet | ping -c 3 8.8.8.8 | 100% packet loss | ✗ BLOCKED |

Level 2 can only talk to the PLC. No internet, no IT access.

# Demo — Level 3: OT Operations

## What to say:

"HISTORIAN01 is on Level 3. It subscribes to MQTT data from the IoT Gateway. But it must never reach the PLC directly — that would bypass the segmentation."

## Machine: HISTORIAN01 (10.10.30.20) — Terminal (SSH)

| What we test | Command | Expected | Result |
| --- | --- | --- | --- |
| Historian subscribes to MQTT broker | nc -zv -w 3 10.10.20.10 1883 | succeeded | ✓ PASS |
| Historian cannot reach PLC directly | nc -zv -w 3 10.10.10.10 502 | timed out | ✗ BLOCKED |

Level 3 gets data through the proper channel — MQTT via Level 2. Direct PLC access is blocked.

# Demo — Level 3.5: DMZ

## What to say:

"The DMZ hosts our public company website. It is accessible from the internet but must never reach the OT network. I will show both — the website loading and the DMZ being blocked from the PLC."

## Step 1 — Open browser:

https://t3textile.serveousercontent.com

Show the website loading to the professor.

## Step 2 — Machine: REVERSE-PROXY (10.10.35.20) — Terminal

| What we test | Command | Expected | Result |
| --- | --- | --- | --- |
| Website accessible from internet | Open in browser | Loads | ✓ PASS |
| DMZ cannot reach PLC | nc -zv -w 3 10.10.10.10 502 | timed out | ✗ BLOCKED |

Even if the web server is hacked, the attacker cannot reach industrial systems.

# Demo — Level 4: Enterprise IT

## What to say:

"WIN-ADMIN01 is on Level 4 — the IT network. IT staff can read factory data via Grafana but must never reach the PLC directly."

## Machine: WIN-ADMIN01 (10.10.40.50) — PowerShell

| What we test | Command | Expected | Result |
| --- | --- | --- | --- |
| IT can read Grafana dashboard | Test-NetConnection -ComputerName 10.10.30.20 -Port 3000 | True | ✓ PASS |
| IT cannot reach PLC | Test-NetConnection -ComputerName 10.10.10.10 -Port 502 | False/Timeout | ✗ BLOCKED |

IT has visibility into OT data through Grafana only. Direct PLC access is completely blocked.

# Closing Statement

"Every level has been tested live. The Purdue Model segmentation is fully enforced — each level can only communicate with what the policy allows. A compromise at any level cannot directly affect a more sensitive level."